Punch Bag

A 1930's  punch bag was required for this production which was proving difficult and expensive to buy.  I decided to make one.

I found a  image on the  internet and printed it out at a suitable size that a teardrop segment could be used as a template.   I decided to make a paper version of the punch bag so that I could check that my idea would work and so that I could get it passed by the designer and the director before I cut out the leather fabric.

We used red/brown leatherette from Mandors(approx £10 per metre)

As the segment I was using as a template was not symmetrical I cut out three pieces with the front of the template facing up and three with it facing down leaving a seam allowance of 5cm.  I then pinned them together and cut a circle of fabric with a radius approximately one third bigger than the radius of the top piece.  Using the industrial sewing machine in the props department I sewed the teardrop  shapes together, right-sides together, leaving a 10cm gap in the top of one seam.  I then attached the bottom circular piece to the top section to create the basic punch bag shape.  I created a loop of black cotton tape and sewed it to the top of the punch bag.  I turned the punch bag right side out through the hole in the side seam.

Using a belt hole punch I punched 3 pairs of holes either side of the open seam and re-inforced them with metal rings.

I poured 4 bags of rice(Tesco Value) into the punch bag and filled up the remaining space with cushion stuffing(Primark cushion).  I threaded a black walking boot lace through the re-inforced holes and pulled them tight to seal.