Scenic Art Department

Freezer paper

stencil

project

- freezer paper makes an excellent single-use stencil

- it works best on unpainted cotton-rich fabrics

- it adheres easily, blocking bleeding and leaves no sticky residue  when removed

- use it shiny side facing the fabric

- Photocopy your design   1:1 scale

- Draw in your “bridges” to all “islands” ALLOWING THE CUT STENCIL TO BE TRANSFRRED TO YOUR FABRIC

- tAPE THE PHOTOCOPY TO THE NON SHINY SIDE

- Use the photocopy as a template and cut THROUGH BOTH THE PHOTOCoPY AND FREeZER PAPER WITH A SCALPEL

- IRON YOUR FABRIC FLAT

- PUT A FEW LAYERS OF NEWSPAPER UNDER THE top layer of FABRIC TO STOP BLEEDING through to the back

- FIND THE CENTRE OF YOUR IMAGE by CAREFULLY folding the image IN HALF

- measure THE CENTRE OF YOUR T-SHIRT

- WHEN YOU ARE HAPPY WITH THE PLACEMENT IRON THE FREEZER PAPER TO THE FABRIC

- USE A STAMPING MOTION TO NOT DISTURB ANY SMALLER CUTS

- CAREFULLY CUT AWAY YOUR BRIDGES

- tAPE A PROTECTIVE LAYER OF NEWSPAPER AROUND THE STENCIL COVERING ALL of the EXPOSED FABRIC

- use screenprinting dye or add textile medium to any acrylic paint to make your own colours

- use a piece of scrap foam or a brush to dab the paint onto the exposed fabric

- use a “dry” technique to minimise any “bleeding” under the stencil

- Be firm and even, leaving no part unpainted

- when you have covered all of the stencil dry it with a hairdryer

- when it is fully dry remove the newspaper before carefully removing the freezer paper

- make sure your hands are clean and dry

- a scalpel might be useful to weed out the “islands”

- IRON THE image USING A CLEAN PIECE OF PAPER FOR 2 MINUTES. THIS WILL SET IT ALLOWING IT TO BE WASHED.

- sTAND BACK AND ADMIRE YOUR WORK